Grouping of Use Cases

1 Certification, Sale and Service of vehicle test equipment

1.1 Initial installation of the workshop network

1.2 Certification of vehicle test equipment

1.3 Introduction of new vehicle test equipment in a workshop

1.4 Replacement of vehicle test equipment

1.5 Temporary use of supplementary vehicle test equipment in a PTI centre

1.6 Collect information on the equipment installed and check if they are homologated and if maintenance was done properly (quality management by the test centre)

2 Performing Tasks

2.1 Manage PTI

2.1.1 Retrieve vehicle specific, PTI relevant data

2.1.2 Perform PTI

2.1.3 Interchange communication between PTI test centre and the Vehicle Information Platform

consider adding strict locking mechanism as for PTI in the Netherland

2.1.4 Perform offline PTI

2.2 Live Data Streaming

2.2.1 Generic live data streaming

2.2.2 Event triggering (e.g. brake tester commands a camera to take the picture)

2.3 Vehicle troubleshooting and repair

2.3.1 Connection to dealer management systems

2.4 Roadside inspection

Use Case 1.1 - Initial installation of the workshop network

| ID/Name | 1.1 / Initial installation of the workshop network |
| --- | --- |
| Actor |  |
| Stakeholder |  |
| Goal |  |
| Non-Goal |  |
| Use case input |  |
| Use case output |  |
| Brief description |  |
| Classification |  |

Use Case 1.2 - Certification of vehicle test equipment

| ID/Name | 1.2 / Certification of vehicle test equipment |
| --- | --- |
| Actor |  |
| Stakeholder |  |
| Goal | Ref. COM 2012-380 proposal, Article 11 and Annex V |
| Non-Goal |  |
| Use case input |  |
| Use case output |  |
| Brief description |  |
| Classification |  |

Use Case 1.3 - Introduction of new vehicle test equipment in a workshop

| ID/Name | 1.3 / Introduction of new vehicle test equipment in a workshop or PTI centre |
| --- | --- |
| Actor |  |
| Stakeholder |  |
| Goal | Ref. COM 2012-380 proposal, Article 11 and Annex V |
| Non-Goal |  |
| Use case input |  |
| Use case output |  |
| Brief description |  |
| Classification |  |

Use Case 1.4 - Replacement of vehicle test equipment

| ID/Name | 1.4 / Replacement of vehicle test equipment |
| --- | --- |
| Actor |  |
| Stakeholder |  |
| Goal | Ref. COM 2012-380 proposal, Article 11 and Annex V |
| Non-Goal |  |
| Use case input |  |
| Use case output |  |
| Brief description |  |
| Classification |  |

Use Case 1.5 - Temporary use of supplementary vehicle test equipment in a workshop

| ID/Name | 1.5 / Temporary use of supplementary vehicle test equipment in a PTI centre |
| --- | --- |
| Actor |  |
| Stakeholder |  |
| Goal | Ref. COM 2012-380 proposal, Article 11 and Annex V |
| Non-Goal |  |
| Use case input |  |
| Use case output |  |
| Brief description |  |
| Classification |  |

2 Performing Tasks

2.1 Manage PTI

Use Case 2.1.1 - Retrieve vehicle specific, PTI relevant data

| ID/Name | 2.1.1 / Retrieve vehicle specific, PTI relevant data |
| --- | --- |
| Actor |  |
| Stakeholder |  |
| Goal | Ref. COM 2012-380 proposal, Preamble points (11) and (25), Article 4 point 3 |
| Non-Goal |  |
| Use case input |  |
| Use case output |  |
| Brief description |  |
| Classification |  |

Use Case 2.1.2 - Perform PTI

| ID/Name | 2.1.2 / Perform PTI |
| --- | --- |
| Actor | Worker in a PTI test centre |
| Stakeholder | Worker, manager of the test centre, vehicle owner, legislators |
| Goal | Perform the required procedure without the need of doing other tasks (e.g. check vehicle specifics) to get the required information. The result is secured and available in the central database. Integrate existing implementations Ref. COM 2012-380 proposal, Article 6 and Annex II |
| Non-Goal | - Invoice |
| Use case input | - Certified test equipment information - Operator and test centre data - Vehicle data - Country and vehicle specific PTI information according to country-specific legislation (similar to ISO 18541-1 UC 5.1.4); in the future this will include data for testing ECSS (see UC 2.1.1) |
| Use case output | Official test result (vehicle roadworthiness) Results of each test Secured test certificate |
| Brief description | - The owner of the vehicle arrives to the test centre - The person at the reception enters the registration information and the vehicle type into the system - The worker fetches the vehicle and performs the required measurements with the required values. This information is provided by the measurement device. To be defined: Responsibility to handle the correct limits. - After the test is finished the measurement device sends the result to our new system in the test centre. - The result contains: vehicle data, measurement data, official test result, test equipment data, test centre and operator data. Variations are possible according to the legislation of each Member State - The system of the test centre delivers the result to the defined authority. - A copy of the official test result and measurement data is printed out for the owner of the vehicle [TBD: this may be out of scope] |
| Classification | Mandatory |

Use Case 2.1.3 - Interchange communication between PTI test centre and the Vehicle Information Platform

| ID/Name | 2.1.3.1 / Retrieve a vehicle identification |
| --- | --- |
| Actor | PTI test centre |
| Stakeholder | National administration, legislators, PTI network |
| Goal | Grab from the national vehicle registration data base information about the vehicle on which PTI has to be performed. |
| Non-Goal | - Grab from national data base about stolen vehicle status |
| Use case input | - Plate number or alternative vehicle id, identification of the demander |
| Use case output | Date of the first registration Date of the registration document Brand / Marque (D.1) National vehicle approval n° (D.2) CNIT n° (D.2.1) Commercial vehicle name (D.3) Maximum technical vehicle available weight (F.1) Maximum weight in use int the member state (F.2) Maximum weight in use int the member state (F.3) Weight of the vehicle with the body and the hooking system (G) Vehicle weight empty (G.1) – Vehicle category (CE) (J) National type (J.1) CE body (J.2) Body (J.3) National vehicle approval n° per type type (K) Engine size (P.1) Engine power (P.2) Type of energy (Petrol, Diesel, LPG …) (P.3) Engine taxe category (P.6) Ratio Power vs weight (Q) Number of seats (S.1) Number of standing passengers (S.2) Level of noise (U.1) Engine speed (U.2) CO2 (V.7) Environnemental CE vehicle category (V.9) Special mention ; value 1, value 2 Owner information Error code : if data are not matching (various sub use case) |
| Brief description | Request is defined as follows by use of: - PTI station id n° + security code - Request n° (for tracking of the request) - Status of vehicle’s document - Date and time of the request - Vehicle Id: - National registration n° or - Vehicle plate n° or - n° from the registration vehicle document |
| Classification | Mandatory |

| ID/Name | 2.1.3.2 / Exploitation of the vehicle identification data |
| --- | --- |
| Actor | PTI test centre |
| Stakeholder | National administration, legislators, PTI network |
| Goal | Feed back in case of any mismatched data between the data set previously get from the registration data base and the really seen by the controler |
| Non-Goal | - Error treatment and data mismatching fixing |
| Use case input | - Data from the previous request and data seen by the controller |
| Use case output | Date of the first registration: New value, error code Date of the registration document : New value, error code Brand / Marque (D.1) : New value, error code National vehicle approval n° (D.2) : New value, error code Sérial n°: New value, error code CNIT n° (D.2.1) : : New value, error code Commercial vehicle name : New value (D.3) Maximum technical vehicle available weight (F.1) : New value, error code Maximum weight in use int the member state F.2): New value, error code Maximum weight in use int the member state (F.3): New value, error code Weight of the vehicle with the body and the hooking system (G) : New value, error code Vehicle weight empty (G.1) : New value, error code Vehicle category (CE) (J) : New value, error code National type (J.1): New value, error code CE body (J.2): New value, error code Body (J.3) : New value, error code National vehicle approval n° per type type (K) : New value, error code Type of energy (Petrol, Diesel, LPG …) (P.3) : New value, error code Engine taxe category (P.6) : New value, error code Number of seats (S.1) : New value, error code Number of standing passengers (S.2): New value, error code Level of noise (U.1) : New value, error code Engine speed (U.2) : New value, error code Owner information: New value, error code |
| Brief description | From the data set get by the request mechanism: Data treatment in order to verify if the data from the central data base and ethe vehicle data are in accordance. Feed back of the appropriate data value or flag on none modificable values. |
| Classification | Mandatory |

| ID/Name | 2.1.3.3 / Inspection data related vehicle |
| --- | --- |
| Actor | PTI test centre |
| Stakeholder | National administration, legislators, PTI network |
| Goal | Report and related data after the vehicle inspection. |
| Non-Goal | - Value from the test equipment (encrypted and non-encrypted data from the various tests equipment) |
| Use case input | - Data from test equipment - Data from the visual inspection report (portable device report) |
| Use case output | Vehicle identification data: - Date of the first registration - Date of the registration document : - Brand / Marque (D.1) : National vehicle approval n° (D.2) - Serial n - CNIT n° (D.2.1) - Commercial vehicle name - Maximum technical vehicle available weight (F.1) : - Maximum weight in use int the member state F.2): - Maximum weight in use int the member state (F.3): - Weight of the vehicle with the body and the hooking system (G) - Vehicle weight empty (G.1) : - Vehicle category (CE) (J) : - National type (J.1): - CE body (J.2): - Body (J.3) : - National vehicle approval n° per type type (K) : - Type of energy (Petrol, Diesel, LPG …) (P.3) : - Engine taxe category (P.6) - Number of seats (S.1) : - Number of standing passengers (S.2): - Level of noise (U.1) : - Engine speed (U.2) : - Owner information: - Maximum speed - Speed limiter available - Number of children seats - ABR System available - Suspension type - EBS available - Axle which can be set in upper position (not in contact with the road) - Number of tanks - Last tank visit date - Last pressure test of the tank’s date - Last seal test of the tank’s date - Specific authorization to be on the road - N° of this special authorization - Date of the special authorization - EOBD available - Country of the owner of the registration vehicle’s certificate - Address of this owner - Plate number - Engine number - Body number Defaults on the vehicle - Default Codes + remarks Measurement from test equipment - Gas analyser values + equipment identification infos (serial number, brand, model number, producer, SW version, HW version, communication version, approval number of equipment, approval number of equipment for communication…) - Opacimeter values + equipment identification infos (…) - Brake testers values + equipment identification infos (…) - Suspension values + equipment identification infos (…) - Side slip values + equipment identification infos (…) - EOBD tester values + equipment identification infos (…) - Headlight beam tester values + equipment identification infos (…) - Decelometer values - Date& date of Gas analyser test - Date& date of Opacimeter test - Date& date of Brake testers test - Date& date of Suspension test - Date& date of Side slip test - Date& date of EOBD tester test - Date& date of Headlight beam tester test - Date& date Decelometer test - File content of Gas analyser test (encrypted and non-encrypted data) - File content of Opacimeter test (encrypted and non-encrypted data) - File content of Brake testers test (encrypted and non-encrypted data) - File content of Suspension test (encrypted and non-encrypted data) - File content of Side slip test (encrypted and non-encrypted data) - File content of EOBD tester test (encrypted and non-encrypted data) - File content of Headlight beam tester test (encrypted and non-encrypted data) - Date& date of the inspection - ID of the inspector - Certificate n° of the inspector - Limit date of the certificate of the inspector - Id of the PTI station - Approval n° of the PTI station - Type of PTI inspection (full or partial) - N° of the test report printed - N° of the PTI sticker - Vehicle miles stones - Status of the load of the vehicle - Number of default - Number of comments |
| Brief description | Set of data from test equipement Set of data from the visual inspection |
| Classification | Mandatory |

| ID/Name | 2.1.3.4 / Data log about vehicle identification request |
| --- | --- |
| Actor | PTI test centre |
| Stakeholder | National administration, legislators, PTI network |
| Goal | Tracking of the various requests: explanation of request with no action behind. |
| Non-Goal |  |
| Use case input |  |
| Use case output | Daily report - Approval ° of the PTI station - Vehicle plate - Date and time of the request - Result of the request - Related errors codes - Request n° - Reason of no action behind |
| Brief description | Tracking of vehicle identification request |
| Classification | Mandatory |

| ID/Name | 2.1.3.5 / Data log about vehicle inspection report transmission |
| --- | --- |
| Actor | PTI test centre |
| Stakeholder | National administration, legislators, PTI network |
| Goal | Tracking of the various requests: explanation of request with no action behind. |
| Non-Goal |  |
| Use case input |  |
| Use case output | Daily report - Approval ° of the PTI station - Report n° - Date and time of the report transmission - Number of try - Result of the last try - Status of the transmission for the national data base point of view |
| Brief description | Tracking of vehicle inspection report transmission |
| Classification | Mandatory |

| ID/Name | 2.1.3.6 / Data log about vehicle inspector manual data modification |
| --- | --- |
| Actor | PTI test centre |
| Stakeholder | National administration, legislators, PTI network |
| Goal | Tracking of the various requests: explanation of any manual data patching |
| Non-Goal |  |
| Use case input |  |
| Use case output | Daily report - Approval ° of the PTI station - Report n° - Date and time of the report transmission - Nature of modification - Reason of modification - Value before modification - Value after modification |
| Brief description | Tracking of vehicle inspection manual input |
| Classification | Mandatory |

| ID/Name | 2.1.3.7 / Local Data log about data modification related to the PTI station modification (SW, HW, controler) |
| --- | --- |
| Actor | PTI test centre |
| Stakeholder | National administration, legislators, PTI network |
| Goal | Tracking of any configuration, parameter change |
| Non-Goal |  |
| Use case input |  |
| Use case output | Modification data log: - Manual modification on values received from test equipment (before and after + id of the controller) - Manual modification on values received from visual inspection remote sw (before and after + id of the controller) - Time stamps (of begging and end of each control) - Controller approval n° change |
| Brief description | Tracking of vehicle inspection manual input |
| Classification | Mandatory |

| ID/Name | 2.1.3.8 / Secure transmission of test results to the Vehicle Information Platform |
| --- | --- |
| Actor | PTI test centre |
| Stakeholder | Test equipment manufacturers, legislators |
| Goal | Test results are available to the entitled parties and are traceable to the test equipment Information is secured against tampering throughout the process Ref. COM 2012-380 proposal, Preamble points (22) and (24), Article 8 point 5 and Article 15 |
| Non-Goal | - Measurement - Local storage of test results - Data management and access control in the Vehicle Information Platform |
| Use case input | - Test results package (output of UC 2.1) |
| Use case output | Test result available to all entitled parties (e.g. police, insurance companies, etc.) |
| Brief description | - The data management system in the PTI test centre applies a digital signature to the test result package - The signed test result package is sent to the central database (national or European depending on the infrastructure of the Member State), according to a defined secure protocol |
| Classification | Mandatory |

| ID/Name | 2.1.4 / Perform offline PTI |
| --- | --- |
| Actor | Mobile PTI test centre |
| Stakeholder | Test equipment manufacturers, legislators |
| Goal |  |
| Non-Goal |  |
| Use case input |  |
| Use case output |  |
| Brief description |  |
| Classification | Mandatory |

2.2 Live Data Streaming

Use Case 2.2.1 - Live Data Streaming

| ID/Name | 2.2.1 / Generic Live Data Streaming |
| --- | --- |
| Actor | A server device (data source), a client device (data sink) and a network manager. For example a brake testing system (BTS), an ECU diagnostic tester (HU-Adapter) and the Network-Manager (NM) installed in the workshop. |
| Stakeholder | Test equipment manufacturers, workshop office personal and mechanics |
| Goal | Transfer synchronous measurement data between two pieces of equipment (for example brake force, pressure, speed and ECU values). |
| Non-Goal |  |
| Use case input | - Device identifiers of receiver and sender |
| Use case output | Data stream |
| Brief description | 1. Initialisation of operation The client device identifies and connects to the counterpart: central communication server (network manager) or server device. The client defines the payload size and contents, the data rate, and the receiver identifier or broadcast transmission. Servers are required to support a minimum data rate (e.g. 3 readings per second) 2. Start of transmission The client sends a command to start the transmission. Data are sent by the server device regularly at the requested rate. Data are received by the client device or by any device in the network if broadcast transmission was selected. 3. End of transmission The client sends a command to stop the transmission. |
| Classification | Mandatory |

Use Case 2.3 - Vehicle Diagnosis and Repair

| ID/Name | 2.3 / Vehicle troubleshooting and repair |
| --- | --- |
| Actor | A testing system (TS), a customer order system (DMS), a data storage service (DSS) and a network manager (NM) installed in the workshop |
| Stakeholder | Test equipment manufacturers, workshop office personal and mechanics |
| Goal | The DMS transfers customer and vehicle data to the testing system and the testing system sends the results back to the DMS. |
| Non-Goal |  |
| Use case input | - Order, customer and vehicle data |
| Use case output | results of the tests |
| Brief description | 1. Initialisation of operation The Dealer Management System (DMS) is activated and informs the network manager (NM) about its services The Testing System (TS) is activated and informs the NM about its services The DMS receives a list of available TSs from the NM 2. Order placement The operator in the office creates a working order in the DMS with reference to a TS in the workshop. The TS receives a list of customer orders and filters those that can be processed by the TS. Order data are stored in the TS for later selection. 3. Order execution The operator on the TS selects a customer order from the list. The information "current time" and "actual operator" is added to the selected order item. Order data is transferred to the DMS as an update. The DMS changes the order’s status to “commenced” and informs the other TSs to prevent its selection on a different TS. The TS can request and receive additional data about the order and the result of previous tests and measurements on the same vehicle. The required diagnostic, measurement, repair or maintenance operation is carried out on the TS. A report with the results can be printed. The TS sends result data to the data storage service (DSS). Results are associated both with the customer order and with the vehicle (registration or identification number). Two separate records may be sent for the two cases. The TS completes the order item with the end time and brief result (e.g. test pass/fail). Order data is transferred as an update to the DMS. In the DMS, the status of the order item changes to completed and informs the other TSs to prevent the selection of a completed order. The results are stored in the DMS vehicle history. 4. Order conclusion The invoice is printed on the DMS. The DMS deletes the order from the TSs. Exceptions in the sequence are possible: e.g. order cancelation for execution on a different TS. In case of disconnection, order data and results are buffered in the DMS, the NM or the TS for later transmission |
| Classification | Mandatory |

Use Case 2.4 - Roadside inspection

| ID/Name | 2.4 / Roadside Inspection |
| --- | --- |
| Actor | PTI organisation |
| Stakeholder | EU Commission, Member States, Test equipment manufacturers, Certification Authority |
| Goal | Inspection of in-use vehicles (technical periodical inspections "PTI" and roadside inspections "RSI") and vehicle registrations Statistical check of PTI tests In the interests of road safety, environmental protection and fair competition it is important to ensure that commercial vehicles in operation are properly maintained and inspected, so that they maintain their safe traffic performance when circulating within the Union. Ref. 2000/30/EC replaced by 2010/379/EU amended by 2010/47/EU |
| Non-Goal |  |
| Use case input | - Certified test equipment information - Operator and test centre data - Vehicle data - Country and vehicle specific PTI information according to country-specific legislation (similar to ISO 18541-1 UC 5.1.4); in the future this will include data for testing ECSS (see UC 2.1.1) |
| Use case output | Test report (certificate?) |
| Brief description | According to the EU law: 2010/379/EU, Recommendation to 2000/30/EC. Explanatory note by the CITA Secretariat. 2010/47/EU, Amendment to 2000/30/EC. Explanatory note by the CITA Secretariat. 2000/30/EC |
| Classification | Mandatory |