|  | Italy |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement | Motorbikes 3-4 wheels |  |  |  |
| Third roller slip percentage | 16 ÷30 % |  |  |  |
| Minimum axle load measurement | Not specified |  |  |  |
| Minimum higher axle load measurement | ≥ 5000N |  |  |  |
| Axle load accuracy | 0÷40°C ± 30N >1000N ± 3% ≥ 1000N |  |  |  |
| Brake force temperature effect |  |  |  |  |
| Minimum higher brake force measurement | ≥ 3000N |  |  |  |
| Brake force accuracy | 0÷40°C ± 20N >1000N ± 2% ≥ 1000N |  |  |  |
| Brake force temperature effect | 0÷40°C |  |  |  |
| Wheel diameter | ≥ 440 |  |  |  |
| Maximum height above floor surface | Max Vehicle inclination ± 5° |  |  |  |
| Minimum roller diameter mm | ≥ 200 |  |  |  |
| Minimum roller length | ≥400 |  |  |  |
| Maximum distance between roller centres | ≤ 450 |  |  |  |
| Maximum distance between inner / outer edge of roller sets | ≤ 800 ≥ not specified |  |  |  |
| Roller speed | ≥ 5 km/h -20% 3000N |  |  |  |
| Friction / adherence | ≥ 0,6 Wet |  |  |  |
| Calibration weighing should be checked at: | 3 points 1000,2000,3000N |  |  |  |
| Calibration force should be checked at: | 3 points 500,1000,2000N |  |  |  |
| Pedal braking force | ≥ 500N |  |  |  |
| Pedal braking force accuracy | 0÷40°C ± 15N >500N ≤± 3% ≥ 500N |  |  |  |
| Hand braking force | ≥ 300N parking brake |  |  |  |
| Hand braking force accuracy | 0÷40°C ± 10N >300N ≤± 3,3% ≥ 300N |  |  |  |
| Unit |  |  |  |  |

|  |  |  |  | Date |  |
| --- | --- | --- | --- | --- | --- |
| Country |  | Garage Equipment Association |  |  |  |
| Name |  | Position in regard WG6 |  |  |  |
| Phone number |  | Mobile number |  | Company |  |

|  |  |  |  | Other specific | Other specific |
| --- | --- | --- | --- | --- | --- |
| Requirement | Cars and vans ≤3.5t | Trucks > 3.5t | Motorbikes |  |  |
| Third roller slip percentage |  |  |  |  |  |
| Minimum axle load measurement |  |  |  |  |  |
| Minimum higher axle load measurement |  |  |  |  |  |
| Axle load accuracy |  |  |  |  |  |
| Brake force temperature effect |  |  |  |  |  |
| Minimum higher brake force measurement |  |  |  |  |  |
| Brake force accuracy |  |  |  |  |  |
| Brake force temperature effect |  |  |  |  |  |
| Wheel diameter |  |  |  |  |  |
| Maximum height above floor surface |  |  |  |  |  |
| Minimum roller diameter mm |  |  |  |  |  |
| Minimum roller length |  |  |  |  |  |
| Maximum distance between roller centres |  |  |  |  |  |
| Maximum distance between inner / outer edge of roller sets |  |  |  |  |  |
| Roller speed |  |  |  |  |  |
| Friction / adherence |  |  |  |  |  |
| Calibration weighing should be checked at: |  |  |  |  |  |
| Calibration force should be checked at: |  |  |  |  |  |