|  | France | Italy | Italy | Germany | U. K. | Spain |
| --- | --- | --- | --- | --- | --- | --- |
| Requirement | Trucks > 3.5t | Trucks > 3.5t | New std for Trucks > 3.5t draft | Trucks > 3.5t | Bus and Truck (V) | Trucks > 3.5t |
| Third roller slip percentage | 27 to 32% | 16 ÷30 % | 25 ÷30 % Note: the slipping value it is measured between the third roller speed and the roller speed by means a second sensor | Not specified* | 20 to 30 % | 27 to 32% |
| Minimum axle load measurement | 200 daN per wheel | Not specified | Not specified | Not specified | NA | 200 daN per wheel |
| Minimum higher axle load measurement | 8 tons per wheel | ≥ 130000N | Capacity ≥ 150000N on testing ≥ 130000N | Not specified | 13000kg | 8 tons per wheel |
| Axle load accuracy | ±20 daN up to 1000 daN ±2% over 1000 daN | 20 ± 5°C ± 1300N <65000N ≤ 2% ≥ 65000N | 20 ± 5°C ± 1300N >65000N ≤ 2% ≥ 65000N | Not specified* | NA | ±20 daN up to 1000 daN ±2% over 1000 daN |
| Brake force temperature effect |  | 0 ÷40°C ± 325N >65000N ≤ 0,5% ≥ 65000N | -5 ÷45°C ± 330N >65000N ≤ 0,5% ≥ 65000N | Not specified | NA |  |
| Minimum higher brake force measurement | 4000 daN | ≥ 40000N | ≥ 40000N | Not specified | low range - max brake force value in the range 600 to 800 kgf - high range- max brake force value in the range 3500 to 4000 kgf - The ability to measure a maximum value of 3000 kg brake force per wheel when the parking brake is tested in the ‘applied mode’ whereby the parking brake is fully applied prior to the rollers being started. | 4000 daN |
| Brake force accuracy | ±10 daN up to 500 daN ±2% over 500 daN | 20 ± 5°C ± 400N >20000N ± 2% ≥ 20000N | 20 ± 5°C single ± 400N >20000N ≤ 2% ≥ 20000N | Sensors ±1% of nominal value at homologation Display reading ±2% of full scale value ±2% deviation left/right | +/- 3 kgf up to 100 kgf +/- 3 per cent of the true value for all readings above 100 kgf. | ±10 daN up to 500 daN ±2% over 500 daN |
| Brake force temperature effect |  | 0 ÷40°C ± 100N >20000N ≤ 0,5% ≥ 20000N | -5 ÷45°C ± 100N >20000N ≤ 0,5% ≥ 20000N | Not specified | NA |  |
| Wheel diameter | 530 to 1400 mm | Not specified | Not specified | Not specified |  | 530 to 1400 mm |
| Maximum height above floor surface | Not specified | Max Vehicle inclination ± 5° | Max Vehicle inclination ± 5° | Not specified | 100mm | Not specified |
| Minimum roller diameter mm | 200 mm | ≥ 240 | ≥ 240 | 150 mm* | 200mm | 200 mm |
| Minimum roller length | 975 mm | Not specified | Not specified | Not specified |  | 975 mm |
| Maximum distance between roller centres | Not specified | Not specified | Not specified | Not specified | 500mm | Not specified |
| Maximum distance between inner / outer edge of roller sets | 850 mm 2800 mm | ≤ 800 ≥ 2800 | ≤ 800 ≥ 2800 | Not specified | No more than 880mm (inner) Not less than 2600mm (outer) | 850 mm 2800 mm |
| Roller speed | 1,8 < V < 2,5 km/h at any time | ≥ 2 km/h -20% 40000N | ≥ 2 km/h Note: the speed it is checked without load. The motor torque it is checked by means a dynamometer bench | 2,0 km/h at any time “strive for 5,0km/h” | 2 to 5.5 km/H | 1,8 < V < 2,5 km/h at any time |
| Friction / adherence | Min 60% wet condition Min 70% dry condition | ≥ 0,6 Wet | ≥ 0,6 Wet | Min 50% wet condition Min 70% dry condition* | 0.6 tyre co-efficient (Wet) | Min 60% wet condition Min 70% dry condition |
| Calibration weighing should be checked at: | 4 points including 0 and 5000 daN maximum weight per wheel | 3 points 30000,50000,80000N | 3 points 20,50,80% | Not specified* | NA | 4 points including 0 and 5000 daN maximum weight per wheel |
| Calibration force should be checked at: | 4 points including 0 and 90% maximum force | 3 points 10000,20000,30000N | 3 points 20,50,80% | Zero point 30% of maximum force 100% maximum force ±2% Accuracy and Deviation left /right to full scale value. And <5% Deviation compared to the higher value | 0, 100, 200, 400 and 600/800 kgf (LOW) 0,1200/1500,2000/2500 and 3500/4000 kgf (HIGH) | 4 points including 0 and 90% maximum force |
| Unit | daN |  |  |  | Kgf |  |
| Pedal braking force |  | ≥ 800N | ≥ 800N |  |  |  |
| Pedal braking force accuracy |  | 20 ± 5°C ± 400N >20000N ± 2% ≥ 20000N | 20 ± 5°C ± 10N >200N ± 5% ≥ 200N |  |  |  |
| Hand braking force |  | ≥ 800N | ≥ 800N |  |  |  |
| Hand braking force accuracy |  | 20 ± 5°C ± 10N >200N ± 5% ≥ 200N | 0 ± 5°C ± 10N >200N ± 5% ≥ 200N |  |  |  |
| Air pressure |  |  | ≥ 1200 kPa |  |  |  |
| Air pressure accuracy |  |  | 20 ± 5°C ± 10 kPa ≤200 kPa ± ≤ 5% ≥ 200 kPa |  |  |  |
| Air pressure resolution |  |  | ≤10 kPa |  |  |  |
| Air pressure should be checked at: |  |  | At least 200, 400, 800 kPa |  |  |  |

|  |  |  |  | Date |  |
| --- | --- | --- | --- | --- | --- |
| Country |  | Garage Equipment Association |  |  |  |
| Name |  | Position in regard WG6 |  |  |  |
| Phone number |  | Mobile number |  | Company |  |

|  |  |  |  | Other specific | Other specific |
| --- | --- | --- | --- | --- | --- |
| Requirement | Cars and vans ≤3.5t | Trucks > 3.5t | Motorbikes |  |  |
| Third roller slip percentage |  |  |  |  |  |
| Minimum axle load measurement |  |  |  |  |  |
| Minimum higher axle load measurement |  |  |  |  |  |
| Axle load accuracy |  |  |  |  |  |
| Brake force temperature effect |  |  |  |  |  |
| Minimum higher brake force measurement |  |  |  |  |  |
| Brake force accuracy |  |  |  |  |  |
| Brake force temperature effect |  |  |  |  |  |
| Wheel diameter |  |  |  |  |  |
| Maximum height above floor surface |  |  |  |  |  |
| Minimum roller diameter mm |  |  |  |  |  |
| Minimum roller length |  |  |  |  |  |
| Maximum distance between roller centres |  |  |  |  |  |
| Maximum distance between inner / outer edge of roller sets |  |  |  |  |  |
| Roller speed |  |  |  |  |  |
| Friction / adherence |  |  |  |  |  |
| Calibration weighing should be checked at: |  |  |  |  |  |
| Calibration force should be checked at: |  |  |  |  |  |