|  | France | Italie | Germany | U. K. | Spain |
| --- | --- | --- | --- | --- | --- |
| Requirement | Car and vans ≤3.5t | Cars and vans ≤3.5t | Car and vans ≤3.5t | Car and vans ≤3.5t | Car and vans ≤3.5t |
| Third roller slip percentage | 25 to 30% | 16 ÷30 % | Not specified* | 25 to 30% | 25 to 30% |
| Minimum axle load measurement | 150 daN | Not specified | Not specified | 150 daN | 150 daN |
| Minimum higher axle load measurement | 2500 daN | ≥ 25000N | Not specified | 2500 daN | 2500 daN |
| Axle load accuracy | ±19,1 daN up to 150 daN ±1,75% over 150 daN | 20 ± 5°C ± 2% | Not specified* | ±19,1 daN up to 150 daN ±1,75% over 150 daN | ±19,1 daN up to 150 daN ±1,75% over 150 daN |
| Brake force temperature effect | ±0,25% between 0 and 40°C | 0 ÷40°C ± 0,25% | Not specified | ±0,25% between 0 and 40°C | ±0,25% between 0 and 40°C |
| Minimum higher brake force measurement | 750 daN | ≥ 6000N | Not specified | 750 daN | 750 daN |
| Brake force accuracy | ±5 daN up to 286 daN ±1,75% over 286 daN | 0 ± 40°C ± 0,25% | Sensors ±1% of nominal value at homologation Display reading ±2% of full scale value ±2% deviation left/right | ±5 daN up to 286 daN ±1,75% over 286 daN | ±5 daN up to 286 daN ±1,75% over 286 daN |
| Brake force temperature effect | ±0,25% between 0 and 40°C | 0÷40°C | Not specified | ±0,25% between 0 and 40°C | ±0,25% between 0 and 40°C |
| Wheel diameter | Not specified | Not specified | Not specified | Not specified | Not specified |
| Maximum height above floor surface | Not specified | Max Vehicle inclination ± 5° | Not specified | Not specified | Not specified |
| Minimum roller diameter mm | 200 mm | ≥ 150 | 150 mm* | 200 mm | 200 mm |
| Minimum roller length | Not specified | ≥ 600 | Not specified | Not specified | Not specified |
| Maximum distance between roller centres | Not specified | Not specified | Not specified | Not specified | Not specified |
| Maximum distance between inner / outer edge of roller sets | Not specified | ≤ 800 ≥ 2000 | Not specified | Not specified | Not specified |
| Roller speed | 4 < V < 5,5 km/h at any time | ≥ 5 km/h -10% 1000 ÷5000N -20% 5001 ÷6000N brake force | > 2,0 km/h at any time “strive for 5,0km/h” | 4 < V < 5,5 km/h at any time | 4 < V < 5,5 km/h at any time |
| Friction / adherence | Minimum 70% | ≥ 0,6 Wet | Min 50% wet condition Min 70% dry condition* | Minimum 70% | Minimum 70% |
| Calibration weighing should be checked at: | 4 points including 0 and 1000 daN maximum weight per wheel | 3 points axle 2000,6000,10000N 3 points single 1000,3000,5000 | Not specified* | 4 points including 0 and 1000 daN maximum weight per wheel | 4 points including 0 and 1000 daN maximum weight per wheel |
| Calibration force should be checked at: | 4 points including 0 and 90% maximum force | 3 points 1000,3000,5000N | Zeropoint 30% of maximum force 100% maximum force ±2% Accuracy and Deviation left /right to full scale value. and <5% Deviation compared to the higher value | 4 points including 0 and 90% maximum force | 4 points including 0 and 90% maximum force |
| Unit | daN |  |  | kgf |  |
| Pedal braking force |  | ≥ 800N |  |  |  |
| Pedal braking force accuracy |  | 20 ± 5°C ± 1% > 400N ± 2% ≥ 400N |  |  |  |
| Hand braking force |  | ≥ 800N |  |  |  |
| Hand braking force accuracy |  | 20 ± 5°C ± 1% > 400N ± 2% ≥ 400N |  |  |  |