|  | France | Italy | Germany | U. K. | Spain |
| --- | --- | --- | --- | --- | --- |
| Requirement | Motorbikes | Motorbikes | Motorbikes | Motorbikes | Motorbikes |
| Third roller slip percentage | No French specification | 16 ÷30 % | No German specification | 20 to 30 % | No Spanish specification |
| Minimum axle load measurement |  | Not specified |  | NA |  |
| Minimum higher axle load measurement |  | ≥ 5000N |  | 750kg |  |
| Axle load accuracy |  | 0÷40°C ± 20N >1000N ± 2% ≥ 1000N |  | (+/- 3 per cent) |  |
| Brake force temperature effect |  | 0÷40°C |  | NA |  |
| Minimum higher brake force measurement |  | ≥ 3000N |  | 300 kgf |  |
| Brake force accuracy |  | 0÷40°C ± 20N >1000N ± 2% ≥ 1000N |  | +/- 3 kgf up to 100 kgf +/- 3 per cent of the true value for all readings above 100 kgf. |  |
| Brake force temperature effect |  | 0÷40°C |  | NA |  |
| Wheel diameter |  | ≥ 300 |  |  |  |
| Maximum height above floor surface |  | Max Vehicle inclination ± 5° |  | 50mm |  |
| Minimum roller diameter mm |  | ≥ 200 |  | 170mm |  |
| Minimum roller length |  | 300÷400 |  | 250mm (Min) 300mm (Max) |  |
| Maximum distance between roller centres |  | ≤ 400 |  | 400mm |  |
| Maximum distance between inner / outer edge of roller sets |  | Not specified |  | NA |  |
| Roller speed |  | ≥ 5 km/h -20% 3000N |  | 2 to 5.5 km/H |  |
| Friction / adherence |  | ≥ 0,6 Wet |  | 0.6 tyre co-efficient (Wet) |  |
| Calibration weighing should be checked at: |  | 3 points 1000,2000,3000N |  | 0, 50, 100 and 200 kg +(+/- 3 per cent) |  |
| Calibration force should be checked at: |  | 3 points 500,1000,2000N |  | 0, 50, 100, 200 and 300 kgf |  |
| Unit |  |  |  | kgf |  |
| Pedal braking force |  | ≥ 500N |  |  |  |
| Pedal braking force accuracy |  | 0÷40°C ± 15N >500N ≤± 3% ≥ 500N |  |  |  |
| Hand braking force |  | ≥ 300N |  |  |  |
| Hand braking force accuracy |  | 0÷40°C ± 10N >300N ≤± 3,3% ≥ 300N |  |  |  |