|  | U. K. |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement | Vans (3000kg) (VII) |  |  |  |
| Third roller slip percentage | 20 to 30 % |  |  |  |
| Minimum axle load measurement | NA |  |  |  |
| Minimum higher axle load measurement | 3000kg |  |  |  |
| Axle load accuracy | NA |  |  |  |
| Brake force temperature effect | NA |  |  |  |
| Minimum higher brake force measurement | 1250 kgf |  |  |  |
| Brake force accuracy | +/- 3 kgf up to 100 kgf +/- 3 per cent of the true value for all readings above 100 kgf. |  |  |  |
| Brake force temperature effect | NA |  |  |  |
| Wheel diameter |  |  |  |  |
| Maximum height above floor surface | 75mm |  |  |  |
| Minimum roller diameter mm | 180mm |  |  |  |
| Minimum roller length | NA |  |  |  |
| Maximum distance between roller centres | 500mm |  |  |  |
| Maximum distance between inner / outer edge of roller sets | No more than 880mm (inner) Not less than 2600mm (outer) |  |  |  |
| Roller speed | 2 to 5.5 km/H |  |  |  |
| Friction / adherence | 0.6 tyre co-efficient (Wet) |  |  |  |
| Calibration weighing should be checked at: | NA |  |  |  |
| Calibration force should be checked at: | 0, 200, 400, 600/800 and 1200/1500 kgf |  |  |  |
| Unit | kgf |  |  |  |